Beyond the Cleanroom: Atomic Layer Deposition as a Molecular Toolkit for Batteries, Plastics, and Nature-Inspired Materials

클린룸을 넘어서: 배터리, 플라스틱, 그리고 자연을 설계하는 원자층증착(ALD)

Seung-Mo Lee1,2*

1Korea Institute of Machinery and Materials (KIMM), Republic of Korea

2University of Science and Technology (UST), Republic of Korea

E-mail address: sm.lee@kimm.re.kr

What do a memory chip, a lithium-ion battery, a plastic film, and a spider’s thread have in common? All of them can be re-engineered one atomic layer at a time. Atomic layer deposition (ALD) grows pinhole-free films with sub-nanometer thickness control on almost any surface, a capability that has made it indispensable in semiconductor manufacturing. Yet the self-limiting surface chemistry at its heart is far too powerful to stay inside the cleanroom.

In this talk, I will take ALD on three “detours.” First, into rechargeable batteries: conformal oxide nanolayers wrapped around electrode particles and porous architectures suppress parasitic interfacial reactions and buffer chemomechanical stress, mitigating the microcracking that drives capacity fade. Second, into polymers: in vapor phase infiltration (VPI), an ALD-derived process, precursors diffuse into and react inside a polymer matrix rather than stopping at its surface, forming organic–inorganic hybrids. The result can be striking, as shown by spider silk that became markedly tougher after metal-oxide infiltration [1], and commodity plastics can likewise gain stiffness, toughness, and stability. Third, into nature: nacre combines strength and fracture toughness through a brick-and-mortar architecture reinforced by molecular-scale inorganic components, a design principle that VPI can emulate at the molecular level.

Along the way, I will discuss the practical side of ALD as a nanomanufacturing tool, including precursor–substrate chemistry, coating powders, fibers, and soft materials, and the remaining challenges in throughput and scale-up [2]. Together, these stories converge on one message: ALD is no longer a niche tool for chipmakers but a versatile platform for designing materials and their mechanical behavior, from the molecule up.

References

- S.-M. Lee et al., Science 324, 488 (2009).

- S. M. George, Chem. Rev. 110, 111 (2010).